Lab Manual

Name : Iman Ishar

Roll No : 2410040305-010

Degree : BS Computer Science

Subject : Information security lab

Semester : Third

Submitted to                                              Mam Ayesha

Grand Asian University Sialkoat

First drag router 2911 then pc

Join them together with console cable

Click on the PC then select Desktop tab then open Terminal.

Then click ok button

You’ll see Router>

Type Router> ? and press Enter

Then you’ll see commands

Type Router> show running-config and press Enter

Now type these commands and enter then one by one

Router> enable

Prompt changes to:

Router#

Router# show running-config

Router# configure terminal

Router(config)# hostname R1

R1(config)#

now set password and encrypt it

R1(config)# enable password cisco123

R1(config)# enable secret Secure@123

Now set console password and enter these commands

R1(config)# line console 0

R1(config-line)# password console@123

R1(config-line)# login

R1(config-line)# exit

Now type these

R1(config)# username admin privilege 15 secret Admin@123

R1(config)# username user privilege 1 secret User@123

This allows login with username/password instead of only console password.

Now Configure Console to Use Local Login

R1(config)# line console 0

R1(config-line)# login local

R1(config-line)# exit

R1# copy running-config startup-config

Destination filename [startup-config]? (press Enter)

Now save this

R1# write memory

R1# exit